SECTION 01 73 29

CUTTING AND PATCHING REQUIREMENTS

- GENERAL

      - RELATED DOCUMENTS

        - Construction Drawings, Technical Specifications, Addenda, and general provisions of the Contract, including Contract General Conditions and Supplementary General Conditions and other Division 01 Specification Sections, apply to this Section.

      - SECTION INCLUDES

        - This section specifies administrative and procedural requirements for cutting and patching.

        - Work included in this Section:

          - Cutting and patching not required to be performed as part of the Work specified in other Sections.

          - Cutting and patching existing construction altered or disturbed to accommodate new construction.

          - Cutting and patching existing construction damaged or defaced during new construction as required to restore to existing or better condition at the time of award of Contract.

          - Cutting and patching required to:

            - Install or correct non-coordinated Work.

            - Remove and replace defective and non-conforming Work.

            - Remove samples of installed Work for testing.

          - All concrete sidewalk, driveways, approaches shall be removed and replaced scoreline-to-scoreline.  Partial removal and saw cutting is no allowed.  Contractor shall match existing concrete thickness when replacing any removed section.

        - Refer to other Sections and drawings for specific requirements of the extent and limitations applicable to cutting and patching, demolishing, or altering existing construction of individual parts of the Work.

          - Requirements of this Section also apply to mechanical and electrical installations. (Refer to Division 22, Division 23 and Division 26 Sections for other requirements and limitations applicable to cutting and patching mechanical and electrical installations).

      - SUBMITTALS

        - Cutting and Patching Proposal:  Where approval of procedures for cutting and patching is required before proceeding, submit a proposal describing procedures well in advance of the time cutting and patching will be performed and request approval to proceed.  Include the following information, as applicable, in the proposal:

          - Describe the extent of cutting and patching required and how it is to be performed.

          - Describe anticipated results in terms of changes to existing construction; include changes to structural elements and operating components as well as changes in the building's appearance and other significant visual elements.

          - List products to be used and firms or entities that will perform work.

          - Indicate dates when cutting and patching is to be performed.

          - List utilities that will be disturbed or affected, including those that will be relocated and those that will be temporarily out-of-service.  Indicate how long service will be disrupted.

          - Where cutting and patching involves addition of reinforcement to structural elements, submit details to show how reinforcement is integrated with the original structure.

          - Approval by the Architect to proceed with cutting and patching does not waive the Architect's right to later require complete removal and replacement of a part of the Work found to be unsatisfactory.

          - Effects on University operations and on concurrent operations construction by other contractors.

      - QUALITY ASSURANCE

        - Requirements for Structural Work:  Do not cut and patch structural elements in a manner that would reduce their load-carrying capacity or load-deflection ratio.

          - Obtain approval from the Architect of the cutting and patching proposal before cutting and patching the following structural elements:

Bearing and retaining walls

Structural concrete

Structural steel

Lintels

Timber and primary wood framing

Structural decking

Stair systems

Miscellaneous structural metals

Equipment supports

Piping, ductwork, vessels and equipment

        - Operational and Safety Limitations:  Do not cut and patch operating elements or safety-related components in a manner that would result in reducing their capacity to perform as intended, or result in increased maintenance, or decreased operational life or safety.

          - Obtain approval of the cutting and patching proposal before cutting and patching the following operating elements or safety-related systems:

Primary operational systems and equipment

Air or smoke barriers

Water, moisture, or vapor barriers

Membranes and flashings

Fire protection systems

Noise and vibration control elements and systems

Control systems

Communication systems

Electrical wiring systems

        - Visual Requirements:  Do not cut and patch construction exposed on the exterior or in occupied spaces, in a manner that would, in the Architect's opinion, reduce the building's aesthetic qualities, or result in visual evidence of cutting and patching. Remove and replace work cut and patched in a visually unsatisfactory manner.

        - If possible retain the original installer or fabricator throughout construction phases to cut and patch the following categories of exposed work, or if it is not possible to engage the original installer or fabricator, Contractor shall engage another recognized experienced and specialized firm:

Concrete finishes

Masonry

Stucco and ornamental plaster

Acoustical ceilings

Painting

Wall covering

HVAC enclosures, cabinets or covers

- PRODUCTS

      - MATERIALS

        - Use materials that are identical to existing materials unless not available. If identical materials are not available or cannot be used where exposed surfaces are involved, use materials that match existing adjacent surfaces to the fullest extent possible with regard to visual effect. BEFORE PROCEEDING CONTRACTOR SHALL OBTAIN APPROVAL OF THE ARCHITECT.

        - Use materials whose installed performance will equal or surpass that of existing materials.

- EXECUTION

      - INSPECTION

        - Before cutting existing surfaces, examine surfaces to be cut and patched and conditions under which cutting and patching is to be performed.  Take corrective action before proceeding, if unsafe or unsatisfactory conditions are encountered.  Inspect existing conditions prior to commencing Work, including elements subject to damage or movement during cutting and patching.

          - Before proceeding, meet at the site with parties involved in cutting and patching, including asbestos abatement, mechanical and electrical trades.  Review areas of potential interference and conflict. Coordinate procedures and resolve potential conflicts before proceeding.

          - Beginning of cutting or patching shall be interpreted to mean that existing conditions were found by Contractor to be acceptable.

          - After uncovering existing Work, Contractor shall inspect conditions affecting proper accomplishment of Work.

      - PREPARATION

        - Temporary Support:  Provide temporary support of Work to be cut where required.

        - Protection:  Protect existing construction during cutting and patching to prevent damage.  Provide protection from adverse weather conditions for portions of the Project that might be exposed during cutting and patching operations.

        - Avoid interference with use of adjoining areas or interruption of free passage to adjoining areas.

        - Take all precautions necessary to avoid cutting existing pipe, conduit or ductwork serving the building, but scheduled to be removed or relocated until provisions have been made to bypass them.

      - PERFORMANCE

        - General

          - Employ skilled workers to perform cutting and patching. Proceed 	with cutting and patching at the earliest feasible time and complete without delay.

          - Cut existing construction to provide for installation of other components or performance of other construction activities and the subsequent fitting and patching required to restore surfaces to their original condition.

        - Cutting

          - Cut existing construction using methods least likely to damage elements to be retained or adjoining construction.  Where possible review proposed procedures with the original installer; comply with the original installer's recommendations.

          - In general, where cutting is required use hand or small power tools designed for sawing or grinding, not hammering and chopping.  Cut holes and slots neatly to size required with minimum disturbance of adjacent surfaces. Temporarily cover openings when not in use.

          - To avoid marring existing finished surfaces, cut or drill from the exposed or finished side into concealed surfaces.

          - Cut through concrete and masonry using a cutting machine such as carborundum saw or diamond core drill.

          - By-pass utility services such as pipe or conduit, before cutting, where services are shown or required to be removed, relocated or abandoned. Cut-off pipe or conduit in walls or partitions to be removed.  Cap, valve or plug and seal the remaining portion of pipe or conduit to prevent entrance of moisture or other foreign matter after by-passing and cutting.

          - Provide fire-safe seals to maintain fire rating at all penetrations.

        - Patching

          - Patch with durable seams that are as invisible as possible.  Comply with specified tolerances.

          - Where feasible, inspect and test patched areas to demonstrate integrity of the installation.

          - Restore exposed finishes of patched areas and extend finish restoration into retained adjoining construction in a manner that will eliminate evidence of patching and refinishing.

          - Where removal of walls or partitions extends one finished area into another, patch and repair floor and wall surfaces in the new space to provide an even surface of uniform color and appearance. Remove existing floor and wall coverings and replace with new materials if necessary to achieve uniform color and appearance.

          - Where patching occurs in a smooth painted surface, extend final paint coat over entire unbroken wall section containing the patch, after the patched area has received primer and second coat.

          - Patch, repair or re-hang existing ceilings as necessary to provide an even plane surface of uniform appearance.

          - Replace concrete walkways to nearest construction joint. Any required repair to a portion of a walkway panel shall require full replacement of said panel from joint to joint in both the north-south and east-west direction.

        - Plaster Installation:  Comply with manufacturer's instructions and install thickness and coats as indicated.

      - CLEANING

        - Thoroughly clean areas and spaces where cutting and patching is performed or used as access. Remove completely paint, mortar, oils, putty and items of similar nature. Thoroughly clean piping, conduit and similar features before painting or other finishing is applied. Restore damaged pipe covering to its original condition.

END OF SECTION